formal hazard assessment

| ## TITLE: Core Line Installation |  |  |  | ## FHA #: |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Positions Considered: |  |  | Assessment Team: |  |  |  | Date Created: |  |  |  |  |
|  |  |  |  |  |  |  | Revision #: |  |  |  |  |
|  |  |  |  |  |  |  | Last Revision Date: |  |  |  |  |
| Task | Health | Safety | Hazards Associated with Task Identify hazards (as applicable) presented by; operations, equipment, vehicles, materials, products, working conditions and environment. | Raw Risk Rating |  |  | Controls Consider the Hierarchy of Controls. |  | Residual Risk Rating |  |  |
|  |  |  |  | Severity | Probability | Overall Risk Rating |  |  | Severity | Probability | Overall Risk Rating |
| Planning and Preparation |  |  | Misplacement of core line, conduit, boxes or sleeves |  |  |  | Eng: Provide an accurate and marked deck layout Adm: Conduct pre-task meetings to review plans and drawings PPE: |  |  |  |  |
|  |  |  | Unclear or conflicting specifications |  |  |  | Eng: Use updated, approved plans and schematics Adm: Verify plans with the site supervisor before work begins PPE: N/A |  |  |  |  |
| Walking on Rebar |  | x | Trips, slips, or falls on uneven rebar | 3 | 3 | 9 | Eng: Use rebar caps and anti-slip coatings where possible Adm: Enforce strict housekeeping to remove obstructions PPE: All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 2 | 2 | 4 |
|  |  | x | Rebar impalement from a fall | 4 | 3 | 12 | Eng: Install rebar caps or protective barriers over exposed ends Adm: Train workers on proper rebar navigation techniques PPE: All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 2 | 2 | 4 |
| Measuring and Marking Locations |  | x | Slips or trips from tools and materials | 2 | 3 | 6 | Eng: Provide tool organization solutions Adm: Enforce housekeeping policies on the deck PPE: All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 2 | 2 | 4 |
|  | x |  | Prolonged exposure to sun or sever weather | 3 | 3 | 9 | Eng: Provide shaded work areas or temporary shelters Adm: Schedule breaks during extreme weather conditions PPE: Wide-brimmed hats and sunscreen for UV protection; All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 2 | 2 | 4 |
|  | x |  | Strains from repetitive motions (e.g., bending or squatting while measuring) | 2 | 3 | 6 | Eng: Use ergonomic measuring tools and extendable markers Adm: Rotate tasks among workers to reduce repetitive strain PPE: Knee pads or support braces if needed; All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 1 | 2 | 2 |
| Using a Drill to Install Boxes and Sleeves |  | x | Injuries from drill operation | 2 | 3 | 6 | Eng: Use drills with safety features, such as auto shut-off Adm: Train workers on proper drill handling and troubleshooting PPE: All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 2 | 2 | 4 |
|  |  | X | Contact with drill debris or sparks | 2 | 2 | 4 | Eng: Use drills with dust and debris collectors Adm: Limit access to the work area during drilling operations PPE: |  | 2 | 1 | 2 |
| Placing and Securing Core Lines | x |  | Overreaching or awkward postures | 2 | 3 | 6 | Eng: Adm: Train workers to maintain proper body positioning. Provide Micro-breaks PPE: All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 1 | 2 | 2 |
|  |  | x | Injuries from securing tools (e.g., pliers, Tie wire) | 2 | 3 | 6 | Eng: Provide the correct pliers and tie wire Adm: Train workers on safe tool use and techniques PPE: All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 2 | 1 | 2 |
| Lifting Coreline Reels onto Deck and Setting up on Jack Stands | x |  | Strains or sprains from lifting heavy reels | 3 | 3 | 9 | Eng: Provide mechanical lifting aids, such as hoists or cranes Adm: Train workers on proper lifting techniques PPE: Back support belts (if necessary); All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel-toe boots. |  | 2 | 2 | 4 |
|  |  | x | Dropping heavy materials | 4 | 3 | 12 | Eng: Use secure lifting equipment and proper attachment points Adm: Assign team lifting for heavy or awkward loads PPE: All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel toe boots. |  | 2 | 2 | 4 |
|  |  | x | Incorrect setup of jack stands | 3 | 3 | 9 | Eng: Use sturdy, level, and load-rated jack stands Adm: Inspect jack stands for proper condition and placement PPE: N/A |  | 2 | 2 | 4 |
| Pulling Core Lines |  |  | Snagging or tangling of core lines | 1 | 3 | 3 | Eng: Use reels or guides for smooth wire management Adm: Assign team members to monitor pulling activities PPE: |  | 1 | 2 | 2 |
|  | x |  | Injuries from the pulling force | 2 | 3 | 6 | Eng: Provide pulling equipment Adm: Train workers on safe pulling techniques PPE: All required PPE – safety helmet, safety glasses, gloves, hi-visibility, steel toe boots. |  | 2 | 1 | 2 |
| Responding to Emergencies |  | x | Falls or injuries from deck work | 4 | 3 | 12 | Eng: Provide accessible first aid stations on each deck level Adm: Train workers in fall rescue and first aid procedures PPE: Emergency-specific PPE (e.g., gloves) |  | 3 | 2 | 6 |
|  |  | x | Transporting injured workers with limited access to the deck | 4 | 2 | 8 | Eng: Provide deck-level emergency evacuation equipment (e.g., stretcher, pulley system) Adm: Pre-plan emergency access routes and drills specific to deck work PPE: High-visibility vests for rescue personnel |  | 3 | 1 | 3 |

Risk Matrix

Probability

Severity

|  | 1 - Insignificant | 2 - Minor | 3 - Major | 4 - Severe |
| --- | --- | --- | --- | --- |
| 4 - Almost Certain | Medium 4 | High 8 | Extreme 12 | Extreme 16 |
| 3 - Likely | Medium 3 | Medium 6 | High 9 | Extreme 12 |
| 2 - Unlikely | Low 2 | Medium 4 | Medium 6 | High 8 |
| 1 - Rare | Low 1 | Low 2 | Medium 3 | Medium 4 |

Probability pertains to the extent of how likely it is for the risk to occur.

The 4 risk rating levels for this component are:

- Rare – unlikely to happen or has never happened.

- Unlikely – has the possibility to happen or has happened at least once.

- Likely – almost sure to happen or has happened multiple times.

- Almost certain – sure to happen or has happened frequently.

Severity is to determine the level of effects that the hazard could cause.

The 4 risk rating levels for this component are:

- Insignificant – can cause injuries/illnesses requiring first aid or negligible property damage.

- Minor – can cause moderate injuries/illnesses requiring medical aid or moderate property damage.

- Major – can cause irreversible injuries/illnesses requiring long term aid or serious property damage.

- Severe – can result in a fatality or irreversible property damage.

Overall Risk Rating

Severity x Probability = Overall Risk Rating

Low - no further action needed; proper control measures are in place.

Medium – proceed with caution and monitor at regular intervals.

High – must be reviewed and reassessed prior to work proceeding.

Extreme – work must be stopped until adequate controls have reduced hazard to an acceptable level.

formal hazard assessment review Signoff

| FORMAL HAZARD ASSESSMENT: |  |
| --- | --- |
| DEPARTMENT: |  |
| DATE OF REVIEW: |  |

Disclosure: by signing this formal hazard assessment review record I hereby acknowledge that I have participated in the review of this hazard assessment and understand the tasks, hazards, and controls as they written. I agree that I participated in the review of the hazard assessment and understand the engineering, administrative and personal protective equipment that I must use to protect myself from conducting tasks associated with this hazard assessment.

| ATTENDEES |  |  |
| --- | --- | --- |
| PRINT NAME | POSITION | SIGNATURE |